# 5311E	ECOPLUS SYSTEMS FASTLOCK SUSPENDED CEILINGS

## 1.	GENERAL

If you have pre-customised this work section using the "questions and answers" provided as part of the downloading process, it may be necessary to amend some clauses to suit the final project-specific version.

The section must still be checked and customised to suit the project being specified, by removing any other irrelevant details and adding project-specific details and selections.

This section relates to the manufacture, supply and installation of Ecoplus Systems proprietary suspended ceiling system.

It includes:

-	Suspension system

-	Ceiling tiles

-	Grid

-	Perimeter trim profile

-	All elements offered by the manufacturer to complete the system

Modify or extend the above description to suit the project being specified.

This section refers to a ceiling comprising of individual tiles supported on an exposed grid.

### 1.1	RELATED WORK

Refer to ~ for ~

Include cross references to other sections where these contain related work.

### 1.2	ABBREVIATIONS

The following abbreviations are used throughout this part of the specification:

IAB	Integrated acoustic backer

NRC	Noise reduction co-efficient

STC	Sound transmission class

AWCINZ	Association of Wall and Ceiling Industries of New Zealand Inc

Documents

### 1.3	DOCUMENTS

Refer to the general section 1233 REFERENCED DOCUMENTS.  The following documents are specifically referred to in this section:

NZBC C/AS1-AS7	Protection from fire

NZBC C/VM2	Protection from fire

AS/NZS 1170.1	Structural design actions - Permanent, imposed and other actions

NZS 1170.5	Structural design actions - Earthquake actions - New Zealand

AS 1530.2	Methods for fire tests on building materials, components and structures - Test for flammability of materials.

AS/NZS 1530.3	Methods for fire tests on building materials, components and structures - Simultaneous determination of ignitability, flame propagation, heat release and smoke release

AS/NZS 2785	Suspended ceilings - Design and installation

AS 2946	Suspended ceilings, recessed luminaires and air diffusers - Interface requirements for physical compatibility

NZS 4219	Seismic performance of engineering systems in buildings

ISO 354	Acoustics - Measurement of sound absorption in a reverberation room

ISO 5660.1	Reaction-to-fire tests - Heat release, smoke production and mass loss rate - Part 1: Heat release rate (cone calorimeter method)

ISO 11654	Acoustics - Sound absorbers for use in buildings - Rating of sound absorption

ISO 717-1	Acoustics - Rating of sound insulation in buildings and of building elements - Part 1: Airborne sound insulation

ASTM C423	Test method for sound absorption and sound absorption coefficients by the reverberation room method

ASTM E1414	Standard test method for airborne sound attenuation between rooms sharing a common ceiling plenum (two room method)

ASTM C 635	Standard specification for the manufacture, performance and testing of metal suspension systems for acoustical tile and lay-in panel ceilings

ASTM C636	Standard practice for installation of metal ceiling suspension systems for acoustical tile and lay-in panels

Delete from the DOCUMENTS clause any document not cited.  List any additional cited documents.

The following are related documents and if referred to in the work section need to be added to the list of DOCUMENTS:

ISO 11654	Ratings standard for sound absorption

BRANZ BU 369	Fitting tolerances

### 1.4	MANUFACTURER/SUPPLIER DOCUMENTS

Manufacturer's and supplier's documents relating to work in this section are:

Ecoplus Systems data sheets for the selected systems.

Branz Fire Test Report  FH 4847 - Vinyl Tone Plasterboard Ceiling Tile

Branz Fire Test Report FH 4908 - Performance Plus Ultra, Endurance and Economy Plus Fissured Ceiling Tiles

Branz Fire Test Report FH 5198 - Cone Calorimeter Test and NZBC Verification Method C/VM2 Appendix A Performance of Ecoplus Acoustic Plus Range

Manufacturer/supplier contact details

Company:	Ecoplus Systems Limited

Web:	www.ecoplus-systems.com

Email:	sales@ecoplus-systems.com

Telephone:	0800 432 675

It is important to ensure that all personnel on site have access to accurate, up to date technical information on the many products, materials and equipment used on a project.  In most cases individual products are not used in isolation, but form part of a building process.  Also a particular manufacturer's and/or supplier's requirements for handling, storage, preparation, installation, finishing and protection of their product can vary from what might be considered the norm.  Access to technical information can help overcome this potential problem.

### 1.5	WARRANTY - MANUFACTURER/SUPPLIER

Provide a material manufacturer/supplier warranty:

15 years:	For ceiling tiles

15 years:	For suspension components

-	Provide this warranty on the manufacturer/supplier standard form.

-	Commence the warranty from the date of practical completion of the contract works.

Refer to the general section 1237 WARRANTIES for additional requirements.

Modify or expand the clause to suit project or manufacturer/supplier requirements, options include:

-	Change the standard form to be used (check with the manufacturer/supplier, use the general section 1237WA WARRANTY AGREEMENT if required.

-	Commence the warranty from the date of purchase (check with the manufacturer/supplier)

Requirements

### 1.6	NO SUBSTITUTIONS

Substitutions are not permitted to any specified Ecoplus Systems, or associated components and products.

### 1.7	SAMPLE SECTION

Allow to erect a sample section of the suspended ceiling system offered.  Subject to confirmation in writing, the sample section may form part of the completed installation.

Refer to SELECTIONS for location.

Amend or expand this clause as appropriate to the project, including requirements for incorporating other elements - luminaires, diffusers, surrounding surfaces - into the sample section.

### 1.8	INSTALLATION

Minimum installation to AS/NZS 2785 and ASTM C636.  Installation to be only by the manufacturer's accredited installer, using the manufacturer's technical services.  Accredited installers to be members of the Association of wall and ceiling industries (AWCI) of New Zealand.  Provide evidence of experience, listing completed projects of similar size and complexity.

Installation to NZS 4219; with related building services installations complying specifically with clauses 5.9 Ducting, 5.14 Luminaires, and 5.13 Ceiling-suspended equipment and equipment in ceiling voids.

### 1.9	CLEANING

Clean using soft bristled brush, vacuum or a damp cloth or as noted in product data sheet.

### 1.10	SPARES

Provide spare matching ceiling elements in the quantities specified below.  Deliver into a dry store at the site or elsewhere as directed and at agreed times.  Refer to SELECTIONS for quantity.

Ask for packs of tiles.

Compliance information

### 1.11	INFORMATION REQUIRED FOR CODE COMPLIANCE

Provide the following compliance documentation: -

-	Applicators approval certificate from the manufacturer / importer / distributor

-	Manufacturer's, importers or distributors warranty

-	Installer's / applicator's warranty

-	Producer Statement - Construction from the applicator / installer

-	Producer Statement - Construction Review from an acceptable suitably qualified person

-	Other information required by the BCA in the Building Consent Approval documents.

List the Producer Statements - Construction (PS3) required from installers that are required as a condition of carrying out this work.

List also Producer Statements - Construction Review (PS4) required from consultants to be engaged by the Contractor.

Producer Statement - Design (PS1), may be required where certain design work is undertaken by the Contractor or manufacturer / importer / distributor.

Performance

Use these clauses if specifying by performance.  Even when specifying by brand it may still be necessary to include some reference to performance.

### 1.12	LOADING CODE REQUIREMENT

Comply with the requirements of:

-	NZS 1170.5, section 8, Requirements for parts and components.

### 1.13	REACTION TO FIRE PROPERTIES

Reaction to fire properties of the ceiling system must meet the acceptable solutions to all risk groups NZBC C/AS1 - C/AS7.

### 1.14	ECOPLUS PRODUCT FIRE GROUP NUMBERS

The Group Number Classification to NZBC C/AS2-C/AS6, table 4.1, has been determined in accordance with NZBC C/VM2 Appendix A, following testing to ISO 5660.1.

| Product | Group Number |
| --- | --- |
| Ecoplus Acoustic Plus ceiling tile range | 1-S |

### 1.15	CERTIFICATION

Provide:

-	certification of compliance with:

-	NZS 1170.5, section 8, Requirements for parts and components

-	certificates and other evidence that the system offered complies with the standards of performance specified; and

-	a Producer Statement on completion.

### 1.16	ACOUSTIC REQUIREMENTS

Verify material sound absorption performance.  Submit a copy of a full scale sound absorption test report to ISO 354 E200 as tested by Auckland Uni Services or other independent testing authority.  Refer to SELECTIONS for acoustic performance requirements.

### 1.17	ENVIRONMENTAL REQUIREMENTS

Design the ceiling system for use over its expected life without deterioration within the required temperature and humidity range.  Refer to SELECTIONS for details.

If this clause is not used and the SITE CONDITIONS clause cannot be complied with, then allowance needs to be made for a tile and sheet linings that will not sag over the construction period.

### 1.18	LIGHT REFLECTANCE

To ASTM C1477.  Refer to SELECTIONS for reflectance and colour.

Data from selected product technical data sheet

## 2.	PRODUCTS

Select the proprietary system to suit the brief.  Use the PRODUCTS clauses to get the desired finish and required fire and acoustic characteristics.  Use the EXECUTION clauses required for the selected system.

Materials - exposed grid system

### 2.1	FASTLOCK SUSPENDED CEILING GRID

Manufactured in New Zealand by Ecoplus Systems Limited.  Hot-dip galvanized steel elements to ASTM C635 for carrying ceiling panels, light fixtures and air distribution elements and complying with NZS 1170.5, section 8.

Brand:	FasTLock™ Suspended Ceiling Grid

Grid type:	FasTLock™ 24 or FasTLock™ 15, two way exposed

Grid finish/colour:	White

Refer to SELECTIONS for types.

FasTLock 24 is general purpose ceiling grid for square edge and tegular edge tiles, FasTLock 15 is narrow profile ceiling grid for tegular edge tiles.

Confirm the compatibility of recessed lighting, air conditioning unit and ventilation outlets to the ceiling grid.

### 2.2	PERIMETER TRIM

Manufactured by Ecoplus Systems Limited.  Hot-dip galvanized pre-painted steel.

Brand/form:	FasTLock™ Perimeter Trim

Material:	Hot-dip galvanized steel

Finish/colour:	White

Refer to SELECTIONS

Materials - ceiling tiles

### 2.3	CEILING TILES - GENERAL

Brand:	Ecoplus Systems

Edge profile:	to suit grid

Performance:	ASTM C423, ASTM E1414

### 2.4	THERMO ACOUSTIC CEILING PANELS

Ecoplus Systems Acoustic Plus Thermo Acoustic Ceiling Panel, manufactured from a 80kg/m³ glass fibre acoustic absorber, 50mm thick, finished with a composite glass mat acoustic facer.  Tested to ISO 5660.1.  Available in a number of finishes.  Refer to SELECTIONS.

Suitable for open plan offices, call centres, school halls and classrooms, and areas requiring thermal effficiency.

## 3.	EXECUTION

Conditions

### 3.1	CO-ORDINATE SERVICES

Co-ordinate and co-operate with electrical and mechanical work to avoid conflict between suspension members and luminaires, diffusers, pipework and ducting.  Confirm the provision of extra hangers and fixings.

Ensure co-operation with work in and above the ceiling, including the marking of specific ceiling tiles below major access points to above-ceiling services.  Colour coded markings to follow the standards laid down by mechanical and electrical services.

### 3.2	SITE CONDITIONS

Do not begin installation until the building is closed in, fully glazed, the roof watertight, and mechanical and electrical duct work above the ceiling completed.

If fast track construction is used, this may not be possible and non-sag high humidity resistant composite soft fibre tiles must be selected accordingly.

### 3.3	COMPATIBILITY

To AS 2946 for interface requirements for physical compatibility.

### 3.4	RESPONSIBILITY

Ensure that conditions are suitable for the ceiling installation.  Arrange for the programming of the work to suit required practice.

Application

### 3.5	INSTALL

Install the system to AS/NZS 2785 minimum standards and the ceiling manufacturer's requirements.

### 3.6	ACCESSIBILITY

Provide access to the ceiling system and the in ceiling and above ceiling services so that maintenance and removal of any part can be carried out without damage to the ceiling system or panels.

As full access in all areas is clearly impractical, any special requirements for clear access will need to be defined; either by description here, or on the drawings.

### 3.7	PENETRATIONS

Accommodate recessed light fittings, air conditioning outlets and other electrical and/or mechanical services that are fixed to or pass through the ceiling system.  Provide independent support for these as necessary.  Such fittings are not to be supported by the acoustical ceiling panels.

### 3.8	RETURN AIR PLENUM

Provide sufficient air return grilles to reduce filter effect through tiled ceilings, clip down where required.  Consult Ecoplus Systems for appropriate tile selection.

Non foil backed soft fibre and tissue backed perforated panels are not recommended for applications where the ceiling plenum is used as an air return duct due to the filtration effect which occurs leading to dirt build up on the face.  Consult Ecoplus Systems for appropriate tile selection.

### 3.9	PROTECT EXISTING WORK

Protect adjacent existing work from damage during the installation.

Completion

### 3.10	REPLACE

Replace or repair damaged or marked elements.

### 3.11	LEAVE

Leave work to the standard required by following procedures.

### 3.12	REMOVE

Remove debris, unused elements and elements from the site.

### 3.13	CLEAN

Clean soiled or marked units.

## 4.	SELECTIONS

For further details on selections go to www.ecoplus-systems.com.

Substitutions are not permitted to the following, unless stated otherwise.

SELECTIONS is for providing details of the actual selections to be included in the contract works including model numbers, colours and other information necessary to ensure that the correct materials are supplied and installed.

Requirements

### 4.1	SAMPLE SECTION

Location:	~

Size:	~ metres x ~ metres

Performance

Use these performance clauses only where system is being specified by performance; otherwise delete all performance clauses and specify under system.

### 4.2	ACOUSTIC REQUIREMENTS

Practical sound absorption properties to ISO 354 E200, ratings to ISO 11654

| Frequency | 125 Hz | 250 Hz | 500 Hz | 1000 Hz | 2000 Hz | 4000 Hz |
| --- | --- | --- | --- | --- | --- | --- |
| αP | ~ | ~ | ~ | ~ | ~ | ~ |

Sound absorption class:	~

Weighted sound absorption coefficient αW:	~ minimum

Sound reduction to ISO 717-1 Rw:	~ minimum (where applicable)

NRC to ASTM C423-99:	~ minimum

STC to ASTM 413-87:	~ minimum (where applicable)

Specify individual frequency Hz for best results, acoustic data available from selected product technical data sheet or acoustical test report.

Sound absorption class options: A, B, C, D, and E.

### 4.3	ENVIRONMENTAL REQUIREMENTS

Range:	~ - ~°C

Relative humidity:	~ % maximum

If this clause is not used and the SITE CONDITIONS clause cannot be complied with, then allowance needs to be made for a tile that will not sag over the construction period.

### 4.4	REFLECTANCE

Reflectance:	~ % minimum

For (colour):	~

Data from selected product technical data sheet

### 4.5	SCHEDULE

| Area | Product reference | Finish | Sound absorption αW |
| --- | --- | --- | --- |
| ~ | ~ | ~ | ~ |

Identify areas and note product type, finish and acoustical ratings required.

Materials - exposed grid system

### 4.6	GRID SUSPENSION SYSTEM

Location:	~

Brand:	Ecoplus Systems

System:	FasTLock™ Suspended Ceiling Grid

Type:	~

Module:	~mm x ~mm

Grid colour:	~

Options:

Type:	FasTLock ™24 or FasTLock™ 15

Module:	1200mm x 600mm or 600mm x 600mm

Grid colour:	White or black

### 4.7	PERIMETER TRIM

Brand:	Ecoplus Systems

Type:	FasTLock™Perimeter Trim 24mm

Material:	Hot-dipped galvanized steel

Colour:	~

Options:

Colour:	White or black

Schedule item only when a special, non-standard perimeter trim is envisaged.

Materials - ceiling tiles

### 4.8	THERMO ACOUSTIC CEILING PANELS

Location:

Brand/product:	Ecoplus Systems Acoustic Plus Thermo Acoustic Ceiling Panels

Material:	Glass fibre

NRC:	~

Thickness:	50mm

Size/edge:	1195mm x 595mm x 50mm (SQ edge)

Finish:	~

Group number:	1 - S

Options:

NRC:	0.95 αw 0.95

Finish:	White Spray, Mixed, White Tissue, Black Tissue

Accessories

### 4.9	ACCESS HATCHES

Type:	~

Material:	~

Finish/colour:	~

Size:	~mm x ~mm

Access hatches may be required to access M/E services in plenum, size according to need.

Spares

### 4.10	SPARES

Panels:	~

Ask for packs of tiles.