Acoustic Plus: PVC Cleanroom Suspended Ceiling System

Preliminary

Refer to the General Conditions of Contract and to the Special Conditions of this Specification as appropriate. Read this section in conjunction with all other trade sections.

Standards

Comply with the New Zealand Building Code 1992 including all revisions and amendments, Verification Methods where appropriate, and construction principles that are embodied in the Acceptable Solutions.

Comply with all relevant provisions and recommendations of: -

1170.5:2004 (NZS)	Structural design actions – Earthquake actions – New Zealand

2785:2000 (AS/NZS)	Suspended ceilings – Design and installation

4219:2009 (NZS)	Seismic performance of engineering systems in buildings

ASTM C635	Standard specification for the manufacture, performance and testing of metal suspension systems for acoustical tile and lay-in panel ceilings

ASTM C423	Test method for sound absorption and sound absorption coefficients by the reverberation room method

ASTM C636	Standard practice for installation of metal ceiling suspension systems for acoustical tile and lay-in panels

ASTM E1414	Standard test method for airborne sound attenuation between rooms sharing a common ceiling plenum (two room method)

ASTM E413-87	Test method for airborne sound transmission STC

ISO 11654	Weighted sound absorption ratings

ISO 354	Test method for sound absorption by reverberant room method

Scope

Supply and install at 1200mm x 600mm centres or 600mm x 600mm centres, white coloured, Ecoplus FasTLock 24 Suspended Ceiling Grid, and selected Ceiling Panels to the areas identified on the drawings complete with all accessories. All aspects of this work shall be in complete accordance with Ecoplus Technical Literature and Installation Manuals, (check www.ecoplus-systems.com, or call 09 373 1859, for the latest editions), and other relevant product manufacturers' recommendations. 

No substitutions are permitted for Ecoplus FasTLock Suspended Ceiling Grid or Ceiling Panel products.

Early Fire Hazard Properties

Early fire hazard properties of the ceiling system must not exceed the following indices when tested in accordance with AS 1503.
Spread of Flame: 0
Smoke Developed: 3 in exitways, 5 in offices.

Sample Section

Allow to erect a sample section of the suspended ceiling system offered. Subject to confirmation in writing, the sample section may form part of the completed installation.
Location: ¬
Size: ¬m x ¬m.

Ceiling Panel

Acoustic Plus: PVC Cleanroom Ceiling Panels.

Nominal Size: 		(1200mm x 600mm) or (600mm x 600mm)

Thickness:		25mm

Edge:			Square

Pattern #:		(262) or (154) or (991)

Product code:		CLE ¬

Co-operation

Co-operate with other trades to ensure that all preliminary and preparatory works are completed to specification and as shown on the drawings.

Delivery & Handling

Check for damage upon delivery and reject damaged products. Store materials under cover, away from damage on a dry flat surface. Keep dry and protect from damage until use. Use all materials in accordance with the manufacturer's guidelines.

Preparation

The building must be fully enclosed and dry before installation commences. Check that all timber framing, mechanical, electrical, and other services above the suspended ceiling are completed as required before installation.

Workmanship

All installation work shall be carried out by experienced and competent tradesmen familiar with the products specified and installation techniques, to the requirements of AS/NZS 2785, the manufacturer's recommendations and as shown on the drawings. 
Do not use damaged or faulty products.
Installation to be only by the manufacturer's accredited installer, using the manufacturer's technical services. Accredited installers to provide evidence of experience, listing completed projects of similar size and complexity.

Installation

Setout suspended grids to the areas, levels and details shown on the drawings. 
Install suspended grid rails and perimeter trim level, straight and true to line, to the required tolerances. 
All fixing types and suspension wires selected and installed to the manufacturer's requirements.

Accommodate recessed light fittings, air conditioning outlets and other electrical and/or mechanical services that are fixed to or pass through the ceiling system. Provide independent support for these as necessary. Such fittings are not to be supported by the ceiling panels.

Protect adjacent existing work from damage during the installation.

Spares

Panels: Supply¬ cartons of the specified ceiling panel, as spares, to the Client upon completion.

Completion

Check that the suspended grid has been installed correctly to the required setout and is level and true to line. Check for damage and surface marking replace as necessary. 
Issue to the main contractor a copy of the Ecoplus Ceiling Panel maintenance requirements and the Ecoplus Ceiling Panel Warranty. 
Remove all rubbish and waste from the site.